# Let’s Make an Atari 2600 (VCS) Game!

By Darrell Spice, Jr.

# Introduction

For my Atari 2600 Homebrew presentation I've been giving a rundown of the challenges involved in writing an Atari game, namely the limited resources and capabilities of the hardware, as well as the tools (DASM, bB, Stella, Harmony, etc.) and resources (AtariAge, Mini Dig, etc.) that are available to help you.

I've been updating the presentation for each time I give it. On the most recent update, for the 2013 Houston Arcade Expo, I added code for a very simple 2600 program. You can download the source and ROM from my website—colorful.zip.

The code addition went over very well so I've decided to expand upon it for my next presentation, which will be given the weekend of August 16th at the Classic Game Fest in Austin. I decided a very simple game would be the way to go and have worked up a mockup of what it might look like:

The game's going to be called Collect. It's a 1 player game and your objective is to collect as many boxes as you can before the timer runs out. My goals for code are to show:

- How to use TV Type, Select and Reset console switches

- How to use joystick to move player

- How to use the hardware collision detection

- How to use a 2 Line Kernel to draw a reflected playfield with 2 players and 2 missiles

- Sound Effects (timer tick, collected box, end-of-game)

# Step 1: Generate a Stable Display

First things first—head over to MiniDig - Best of Stella and download the Stella Programmer's Guide from the docs page. I've also attached it to my blog entry, but you should still check out what's available over at MiniDig.

The heart of the Atari is the Television Interface Adaptor (TIA). It's the video chip, sound generator, and also handles some of the controller input. As a video chip, the TIA is very unusual. Most video game systems have memory that holds the current state of the display. Their video chip reads that memory and uses that information to generate the display. But not the TIA—memory was very expensive at the time, so the TIA was designed with a handful of registers that contain just the information needed to draw a single scanline. It's up to our program to change those registers in real-time so that each scanline shows what its supposed to. It's also up to our program to generate a "sync signal" that tells the TV when its time to start generating a new image.

Turn to page 2 of the programmer's guide. You'll find the following diagram, which I've slightly modified:

The Horizontal Blank is part of each scanline, so we don't need to worry about generating it. Everything else though is up to us! We need to generate a sync signal over 3 scanlines, after which we need to wait 37 scanlines before we tell TIA to "turn on" the image output. After that we need up update TIA so each of the 192 scanlines that comprise visible portion of the display show what they're supposed to. Once that's done, we "turn off" the image output and wait 30 scanlines before we start all over again.

In the source code, available below, you can see the Main Program Loop which follows the diagram:

Main:

jsr VerticalSync    ; Jump to SubRoutine VerticalSync

jsr VerticalBlank   ; Jump to SubRoutine VerticalBlank

jsr Kernel          ; Jump to SubRoutine Kernel

jsr OverScan        ; Jump to SubRoutine OverScan

jmp Main            ; JuMP to Main

Each of the subroutines handles what's needed, such as this section which generates the sync signal:

VerticalSync:

lda #2      ; LoaD Accumulator with 2 so D1=1

sta WSYNC   ; Wait for SYNC (halts CPU until end of scanline)

sta VSYNC   ; Accumulator D1=1, turns on Vertical Sync signal

sta WSYNC   ; Wait for Sync - halts CPU until end of 1st scanline of VSYNC

sta WSYNC   ; wait until end of 2nd scanline of VSYNC

lda #0      ; LoaD Accumulator with 0 so D1=0

sta WSYNC   ; wait until end of 3rd scanline of VSYNC

sta VSYNC   ; Accumulator D1=0, turns off Vertical Sync signal

rts         ; ReTurn from Subroutine

Currently there's no game logic, so the VerticalBlank just waits for the 37 scanlines to pass:

VerticalBlank:

ldx #37         ; LoaD X with 37

vbLoop:

sta WSYNC       ; Wait for SYNC (halts CPU until end of scanline)

dex             ; DEcrement X by 1

bne vbLoop      ; Branch if Not Equal to 0

rts             ; ReTurn from Subroutine

The Kernel is the section of code that draws the screen:

Kernel:

; turn on the display

sta WSYNC       ; Wait for SYNC (halts CPU until end of scanline)

lda #0          ; LoaD Accumulator with 0 so D1=0

sta VBLANK      ; Accumulator D1=1, turns off Vertical Blank signal (image output on)

; draw the screen

ldx #192        ; Load X with 192

KernelLoop:

sta WSYNC       ; Wait for SYNC (halts CPU until end of scanline)

stx COLUBK      ; STore X into TIA's background color register

dex             ; DEcrement X by 1

bne KernelLoop  ; Branch if Not Equal to 0

rts             ; ReTurn from Subroutine

For this initial build it just changes the background color so we can see that we're generating a stable picture:

Like Vertical Blank, OverScan doesn't have anything to do besides turning off the image output, so it just waits for enough scanlines to pass so that the total scanline count is 262.

OverScan:

sta WSYNC   ; Wait for SYNC (halts CPU until end of scanline)

lda #2      ; LoaD Accumulator with 2 so D1=1

sta VBLANK  ; STore Accumulator to VBLANK, D1=1 turns image output off

ldx #27     ; LoaD X with 27

osLoop:

sta WSYNC   ; Wait for SYNC (halts CPU until end of scanline)

dex         ; DEcrement X by 1

bne osLoop  ; Branch if Not Equal to 0

rts         ; ReTurn from Subroutine

# Step 2: Timers

In Step 1, I used loops of sta WSYNC commands to delay the program so that Vertical Blank and OverScan would last for the proper duration. That method works fine when all we want to do is generate a static display, but as soon as we start to add game logic that won't work out so well.

The problem with the game logic is there will be so many different paths the code can take that it is nearly impossible for us to know how long the code ran, and thus we won't know how many scanlines we need to delay before the next section of code can run. As an example, if the player isn't moving the joystick then none of the "move player" logic will run. If the player is moving the joystick left and up then the "move horizontal" and "move vertical" logic will run. If the player is only holding the joystick left then only the "move horizontal" logic will run.

Fortunately for us, the Atari 2600 contains a RIOT chip. That acronym stands for RAM, Input/Output and Timer. We're interested in the Timer for this update to Collect, we'll look at RAM and I/O in a later update.

First thing I changed was OverScan. The original routine looked like this:

OverScan:

sta WSYNC   ; Wait for SYNC (halts CPU until end of scanline)

lda #2      ; LoaD Accumulator with 2 so D1=1

sta VBLANK  ; STore Accumulator to VBLANK, D1=1 turns image output off

ldx #27     ; LoaD X with 27

osLoop:

sta WSYNC   ; Wait for SYNC (halts CPU until end of scanline)

dex         ; DEcrement X by 1

bne osLoop  ; Branch if Not Equal to 0

rts         ; ReTurn from Subroutine

So what we want to do is set a timer that will go off after 27 scanlines to pass. There's 76 cycles of time per scanline, so we need the timer to go off after 2052 cycles have passed. When we set the timer, we also select how frequently the timer will decrement in value. RIOT has options to decrement the timer every 1, 8, 64 or 1024 cycles.

The timer is set using a single byte, so it can only be set to any value from 0 to 255. As such, we know we can't use decrement every 1 cycle as 2052 is too large. So let's check if decrement every 8 cycles will work:

2052/8 = 256.5

Almost, but 256 won't fit so we're going to have to use the decrement every 64 cycles option. To figure out the initial value to set the timer to, use this equation:

(scanlines * 76) / 64

The new OverScan routine that uses the timer looks like this:

OverScan:

sta WSYNC   ; Wait for SYNC (halts CPU until end of scanline)

lda #2      ; LoaD Accumulator with 2 so D1=1

sta VBLANK  ; STore Accumulator to VBLANK, D1=1 turns image output off

lda #32     ; set timer for 27 scanlines, 32 = ((27 * 76) / 64)

sta TIM64T  ; set timer to go off in 27 scanlines

; game logic will go here

OSwait:

sta WSYNC   ; Wait for SYNC (halts CPU until end of scanline)

lda INTIM   ; Check the timer

bne OSwait  ; Branch if its Not Equal to 0

rts         ; ReTurn from Subroutine

For Vertical Blank we're going to set up the timer a little different. There's time in the Vertical Sync we can utilize, so we'll set the timer there—look for the code using ldx and stx:

VerticalSync:

lda #2      ; LoaD Accumulator with 2 so D1=1

ldx #49     ; LoaD X with 49

sta WSYNC   ; Wait for SYNC (halts CPU until end of scanline)

sta VSYNC   ; Accumulator D1=1, turns on Vertical Sync signal

stx TIM64T  ; set timer to go off in 41 scanlines (49 * 64) / 76

sta WSYNC   ; Wait for Sync - halts CPU until end of 1st scanline of VSYNC

sta WSYNC   ; wait until end of 2nd scanline of VSYNC

lda #0      ; LoaD Accumulator with 0 so D1=0

sta WSYNC   ; wait until end of 3rd scanline of VSYNC

sta VSYNC   ; Accumulator D1=0, turns off Vertical Sync signal

rts         ; ReTurn from Subroutine

We're also going to check the timer in the Kernel section so we can start drawing the screen as soon as it goes off:

Kernel:

sta WSYNC       ; Wait for SYNC (halts CPU until end of scanline)

lda INTIM       ; check the timer

bne Kernel      ; Branch if its Not Equal to 0

; turn on the display

sta VBLANK      ; Accumulator D1=0, turns off Vertical Blank signal (image output on)

; draw the screen

ldx #192        ; Load X with 192

KernelLoop:

sta WSYNC       ; Wait for SYNC (halts CPU until end of scanline)

stx COLUBK      ; STore X into TIA's background color register

dex             ; DEcrement X by 1

bne KernelLoop  ; Branch if Not Equal to 0

rts             ; ReTurn from Subroutine

For the moment, these changes leave Vertical Blank with nothing to do:

VerticalBlank:

rts             ; ReTurn from Subroutine

# Step 3: Score and Timer Display

After getting a stable display, I like to implement the routines for displaying the score. You can see that in the first builds of Frantic, Medieval Mayhem, and Space Rocks. Even though we're not ready to show the player's score, the display is very useful for showing diagnostic information—such as this build of Frantic which uses it to show which sprites are colliding with the player.

To draw the score we're going to use the playfield graphics. The playfield pattern is comprised of 20 bits stored in 3 bytes of the TIA.

- PF0 - only the upper 4 bits are used, and they're output in reverse order

- PF1 - all 8 bits are used, output as you'd expect

- PF2 - all 8 bits are used, output in reverse order

Andrew Davie posted this rather nice diagram in his 2600 Programming For Newbies tutorial which shows how that works:

The 20 bits are either repeated or reflected to draw the 20 bits on the right half of the screen.

For our game, a two digit score and a two digit timer will meet our requirements. We could show a single digit each in PF1 and PF2, but due to the reversed output of PF2 that would mean we'd need to create both normal and mirrored digit graphics. Instead, we're going to create digit graphics that are 3 bits across, which will allow us to show two digits using PF1. You may have seen the graphics in one of my blog entries with the revised mode file for jEdit. Each digit appears twice as we can use a simple mask to get the tens (AND #$F0) and ones (AND #$0F) digits. If we only saved the image as the ones position we'd need to apply 4 shift instructions to create a tens position image.

PF1 is displayed twice on the screen and if we time it correctly we can change the contents of PF1 so that both sides of the screen are different. Andrew Davie posted another handy diagram that shows the timing:

In looking at the diagram we can figure out the update times required for each instance of PF1.

- Left PF1 - need to update on or after cycle 66 of the prior scanline, or on or before cycle 28 of the current scanline

- Right PF1 - need to update on or after cycle 39 and on or before cycle 54 of the current scanline

As mentioned before, the RIOT chip in the Atari 2600 stands for RAM, Input/Output and Timer. For this update we're going to look at the RAM and Input.

To show the score we need to keep track of a few things, so let's allocate some space in RAM:

ORG $80

; Holds 2 digit score, stored as BCD (Binary Coded Decimal)

Score:          ds 1    ; stored in $80

; Holds 2 digit timer, stored as BCD

Timer:          ds 1    ; stored in $81

; Offsets into digit graphic data

DigitOnes:      ds 2    ; stored in $82-$83, DigitOnes = Score, DigitOnes+1 = Timer

DigitTens:      ds 2    ; stored in $84-$85, DigitTens = Score, DigitTens+1 = Timer

; graphic data ready to put into PF1

ScoreGfx:       ds 1    ; stored in $86

TimerGfx:       ds 1    ; stored in $87

; scratch variable

Temp:           ds 1    ; stored in $88

Vertical Blank is now doing a little bit of work:

VerticalBlank:

jsr SetObjectColors

jsr PrepScoreForDisplay

rts             ; ReTurn from Subroutine

When the macro CLEAN_START initialized the Atari's hardware, it set all the colors to black. So we need to set the object colors if we want to see anything. This routine also reads the state of the console switches (via one of RIOT's Input registers) in order to determine if the player selected Color or Black & White:

SetObjectColors:

ldx #3          ; we're going to set 4 colors (0-3)

ldy #3          ; default to the color entries in the table (0-3)

lda SWCHB       ; read the state of the console switches

and #%00001000  ; test state of D3, the TV Type switch

bne SOCloop     ; if D3=1 then use color

ldy #7          ; else use the b&w entries in the table (4-7)

SOCloop:

lda Colors,y    ; get the color or b&w value

sta COLUP0,x    ; and set it

dey             ; decrease Y

dex             ; decrease X

bpl SOCloop     ; Branch PLus (positive)

rts             ; ReTurn from Subroutine

Colors:

.byte $86   ; blue       - goes into COLUP0, color for player0 and missile0

.byte $C6   ; green      - goes into COLUP1, color for player1 and missile1

.byte $46   ; red        - goes into COLUPF, color for playfield and ball

.byte $00   ; black      - goes into COLUBK, color for background

.byte $0E   ; white      - goes into COLUP0, color for player0 and missile0

.byte $06   ; dark grey  - goes into COLUP1, color for player1 and missile1

.byte $0A   ; light grey - goes into COLUPF, color for playfield and ball

.byte $00   ; black      - goes into COLUBK, color for background

The score and timer will be stored using BCD (Binary Coded Decimal) but for now we'll treat and display them as hexadecimal values (that's why the digit graphics are 0-9 then A-F). This routine takes the upper and lower nybble (4 bits) of the Score and Timer and multiplies them by 5 in order to get the offset into the digit graphic data. The 6507 does not have a multiply command, though it does have a shift feature which is equivalent to *2. If a nybble is value X then X * 2 * 2 + X is the same as X * 5:

PrepScoreForDisplay:

; for testing purposes, change the values in Timer and Score

inc Timer       ; INCrement Timer by 1

bne PSFDskip    ; Branch Not Equal to 0

inc Score       ; INCrement Score by 1 if Timer just rolled to 0

PSFDskip

ldx #1          ; use X as the loop counter for PSFDloop

PSFDloop:

lda Score,x     ; LoaD A with Timer(first pass) or Score(second pass)

and #$0F        ; remove the tens digit

sta Temp        ; Store A into Temp

asl             ; Accumulator Shift Left (# * 2)

asl             ; Accumulator Shift Left (# * 4)

adc Temp        ; ADd with Carry value in Temp (# * 5)

sta DigitOnes,x  ; STore A in DigitOnes+1(first pass) or DigitOnes(second pass)

lda Score,x     ; LoaD A with Timer(first pass) or Score(second pass)

and #$F0        ; remove the ones digit

lsr             ; Logical Shift Right (# / 2)

lsr             ; Logical Shift Right (# / 4)

sta Temp        ; Store A into Temp

lsr             ; Logical Shift Right (# / 8)

lsr             ; Logical Shift Right (# / 16)

adc Temp        ; ADd with Carry value in Temp ((# / 16) * 5)

sta DigitTens,x ; STore A in DigitTens+1(first pass) or DigitTens(second pass)

dex             ; DEcrement X by 1

bpl PSFDloop    ; Branch PLus (positive) to PSFDloop

rts             ; ReTurn from Subroutine

The Kernel's been modified so it now uses the data in DigitOnes and DigitTens to update PF1. Each line of graphic data is output twice so the digits are drawn over 10 scanlines which gives them a better appearance than if they had been drawn over 5 scanlines.

ldx #5

ScoreLoop:              ;   43 - cycle after bpl ScoreLoop

ldy DigitTens   ; 3 46 - get the tens digit offset for the Score

lda DigitGfx,y  ; 5 51 -   use it to load the digit graphics

and #$F0        ; 2 53 -   remove the graphics for the ones digit

sta ScoreGfx    ; 3 56 -   and save it

ldy DigitOnes   ; 3 59 - get the ones digit offset for the Score

lda DigitGfx,y  ; 5 64 -   use it to load the digit graphics

and #$0F        ; 2 66 -   remove the graphics for the tens digit

ora ScoreGfx    ; 3 69 -   merge with the tens digit graphics

sta ScoreGfx    ; 3 72 -   and save it

sta WSYNC       ; 3 75 - wait for end of scanline

;---------------------------------------

sta PF1         ; 3  3 - @66-28, update playfield for Score dislay

ldy DigitTens+1 ; 3  6 - get the left digit offset for the Timer

lda DigitGfx,y  ; 5 11 -   use it to load the digit graphics

and #$F0        ; 2 13 -   remove the graphics for the ones digit

sta TimerGfx    ; 3 16 -   and save it

ldy DigitOnes+1 ; 3 19 - get the ones digit offset for the Timer

lda DigitGfx,y  ; 5 24 -   use it to load the digit graphics

and #$0F        ; 2 26 -   remove the graphics for the tens digit

ora TimerGfx    ; 3 29 -   merge with the tens digit graphics

sta TimerGfx    ; 3 32 -   and save it

jsr Sleep12     ;12 44 - waste some cycles

sta PF1         ; 3 47 - @39-54, update playfield for Timer display

ldy ScoreGfx    ; 3 50 - preload for next scanline

sta WSYNC       ; 3 53 - wait for end of scanline

;---------------------------------------

sty PF1         ; 3  3 - @66-28, update playfield for the Score display

inc DigitTens   ; 5  8 - advance for the next line of graphic data

inc DigitTens+1 ; 5 13 - advance for the next line of graphic data

inc DigitOnes   ; 5 18 - advance for the next line of graphic data

inc DigitOnes+1 ; 5 23 - advance for the next line of graphic data

jsr Sleep12     ;12 35 - waste some cycles

dex             ; 2 37 - decrease the loop counter

sta PF1         ; 3 40 - @39-54, update playfield for the Timer display

bne ScoreLoop   ; 2 42 - (3 43) if dex != 0 then branch to ScoreLoop

sta WSYNC       ; 3 45 - wait for end of scanline

;---------------------------------------

stx PF1         ; 3  3 - x = 0, so this blanks out playfield

sta WSYNC       ; 3  6 - wait for end of scanline

Score and Timer:

When TV Type switched to B&W:

You'll notice that the score and timer are different colors even though they're both drawn using the playfield. This is because I've modified Vertical Sync to turn on SCORE mode. SCORE mode tells the TIA to use the color of player0 for the left half of the playfield and the color of player1 for the right half.

VerticalSync:

lda #2      ; LoaD Accumulator with 2 so D1=1

ldx #49     ; LoaD X with 49

sta WSYNC   ; Wait for SYNC (halts CPU until end of scanline)

sta VSYNC   ; Accumulator D1=1, turns on Vertical Sync signal

stx TIM64T  ; set timer to go off in 41 scanlines (49 * 64) / 76

sta CTRLPF  ; D1=1, playfield now in SCORE mode

...

rts         ; ReTurn from Subroutine

# Step 4: 2 Line Kernel

Let's review the TIA Timing diagram from last time:

We used that to determine when we could safely update the playfield data in order to draw the score and timer. For moveable objects (player0, player1, missile0, missile1 and ball) if you update their graphics during the Visible Screen (cycles 23-76) you run the risk of shearing. For something that's moving fast, like the snowball in Stay Frosty 2, shearing may be an acceptable design compromise:

That snowball should be square, but the left edge has sheared due to the ball object being updated mid-scanline.

To prevent shearing we need to update the objects on cycles 0-22. There's a lot of calculations to be done in the kernel to draw just one player. For Collect I'm using DoDraw, which looks like this for drawing player0:

DoDraw0:

lda #HUMAN_HEIGHT-1 ; 2  2 - height of the humanoid graphics, subtract 1 due to starting with 0

dcp HumanDraw       ; 5  7 - Decrement HumanDraw and compare with height

bcs DoDrawGrp0      ; 2  9 - (3 10) if Carry is Set, then humanoid is on current scanline

lda #0              ; 2 11 - otherwise use 0 to turn off player0

.byte $2C           ; 4 15 - $2C = BIT with absolute addressing, trick that

;        causes the lda (HumanPtr),y to be skipped

DoDrawGrp0:                 ;   10 - from bcs DoDrawGrp0

lda (HumanPtr),y    ; 5 15 - load the shape for player0

sta GRP0            ; 3 18 - update player0 to draw Human

That's 18 cycles to draw a single player. One way to make it easier to fit all the code in is to use a 2 Line Kernel (2LK). In a 2LK we update the TIA's registers over 2 scanlines in order to build the display. For Collect, the current routines are updating them like this:

- player0, playfield

- player1, playfield

The actual code looks like this:

ldy #ARENA_HEIGHT   ; 2  7 - the arena will be 180 scanlines (from 0-89)*2

ArenaLoop:                  ;   13 - from bpl ArenaLoop

; continuation of line 2 of the 2LK

; this precalculates data that's used on line 1 of the 2LK

lda #HUMAN_HEIGHT-1 ; 2 15 - height of the humanoid graphics, subtract 1 due to starting with 0

dcp HumanDraw       ; 5 20 - Decrement HumanDraw and compare with height

bcs DoDrawGrp0      ; 2 22 - (3 23) if Carry is Set, then humanoid is on current scanline

lda #0              ; 2 24 - otherwise use 0 to turn off player0

.byte $2C           ; 4 28 - $2C = BIT with absolute addressing, trick that

;        causes the lda (HumanPtr),y to be skipped

DoDrawGrp0:                 ;   23 - from bcs DoDrawGrp0

lda (HumanPtr),y    ; 5 28 - load the shape for player0

sta WSYNC           ; 3 31

;---------------------------------------

; start of line 1 of the 2LK

sta GRP0            ; 3  3 - @ 0-22, update player0 to draw Human

ldx #%11111111      ; 2  5 - playfield pattern for vertical alignment testing

stx PF0             ; 3  8 - @ 0-22

; precalculate data that's needed for line 2 of the 2LK

lda #HUMAN_HEIGHT-1 ; 2 10 - height of the humanoid graphics,

dcp BoxDraw         ; 5 15 - Decrement BoxDraw and compare with height

bcs DoDrawGrp1      ; 2 17 - (3 18) if Carry is Set, then box is on current scanline

lda #0              ; 2 19 - otherwise use 0 to turn off player1

.byte $2C           ; 4 23 - $2C = BIT with absolute addressing, trick that

;        causes the lda (BoxPtr),y to be skipped

DoDrawGrp1:                 ;   18 - from bcs DoDrawGRP1

lda (BoxPtr),y      ; 5 23 - load the shape for the box

sta WSYNC           ; 3 26

;---------------------------------------

; start of line 2 of the 2LK

sta GRP1            ; 3  3 - @0-22, update player1 to draw box

ldx #0              ; 2  5 - PF pattern for alignment testing

stx PF0             ; 3  8 - @0-22

dey                 ; 2 10 - decrease the 2LK loop counter

bpl ArenaLoop       ; 2 12 - (3 13) branch if there's more Arena to draw

If you look at that closely, you'll see I'm splitting DoDraw a bit so that this is how the 2LK works:

- update player0, update playfield, precalc player1 data for next scanline

- update player1, update playfield, precalc player0 data for next scanline

By pre-calculating data during the visible portion of the scanline, we'll have more time during the critical 0-22 cycles for when we add the other objects.

Since we're updating the players on every other scanline, each byte of graphic data is displayed twice (compare the thickness of the humanoid pixels with the red lines drawn with the playfield). Also, the players never line up as they're never updated on the same scanlines:

Closeup:

The designers of the TIA planned for this by adding a Vertical Delay feature to the players and ball (though sadly not the missiles). The TIA registers for this are VDELP0, VDELP1 and VDELBL. For this update to Collect, I've tied the Vertical Delay to the difficulty switches, putting the switch in position A will turn on the delay for that player so we can experiment with how that works. For the next update I'll set the Vertical Delay based on the Y position of the player (this also means the maximum Y value will be double that of this build).

Left Difficulty A, Right Difficulty B so VDELP0 = 1 and VDELP1 = 0. Sprites line up with the same Y:

Closeup:

Left Difficulty B, Right Difficulty A so VDELP0 = 0 and VDELP1 = 1. Sprites line up when player1's Y = player0's Y + 1:

Sprites line up when player1's Y = player0's Y + 1.

Closeup:

The code that preps the data used by DoDraw looks like this:

; HumanDraw = ARENA_HEIGHT + HUMAN_HEIGHT - Y position

lda #(ARENA_HEIGHT + HUMAN_HEIGHT)

sec

sbc ObjectY

sta HumanDraw

; HumanPtr = HumanGfx + HUMAN_HEIGHT - 1 - Y position

lda #<(HumanGfx + HUMAN_HEIGHT - 1)

sec

sbc ObjectY

sta HumanPtr

lda #>(HumanGfx + HUMAN_HEIGHT - 1)

sbc #0

sta HumanPtr+1

; BoxDraw = ARENA_HEIGHT + HUMAN_HEIGHT - Y position

lda #(ARENA_HEIGHT + HUMAN_HEIGHT)

sec

sbc ObjectY+1

sta BoxDraw

; BoxPtr = HumanGfx + HUMAN_HEIGHT - 1 - Y position

lda #<(HumanGfx + HUMAN_HEIGHT - 1)

sec

sbc ObjectY+1

sta BoxPtr

lda #>(HumanGfx + HUMAN_HEIGHT - 1)

sbc #0

sta BoxPtr+1

...

HumanGfx:

.byte %00011100

.byte %00011000

.byte %00011000

.byte %00011000

.byte %01011010

.byte %01011010

.byte %00111100

.byte %00000000

.byte %00011000

.byte %00011000

HUMAN_HEIGHT = * - HumanGfx

The graphics are much easier to see using my mode file for jEdit:

I'm sure some of you are wondering why the human graphics are upside down. If you wanted to loop through something 10 times, you'd normally think to write the code like this:

ldy #0

Loop:

; do some work

iny

cpy #10

bne Loop

But the 6507 does an automatic check for 0 (as well as positive/negative) which lets you save 2 cycles of processing time by eliminating the CPY command:

ldy #10

Loop:

; do some work

dey

bne Loop

Alternatively, if your initial value is less than 128, you can use this:

ldy #(10-1)

Loop:

; do some work

dey

bpl Loop

Making the loop count down instead of up saves 2 cycles, but doing so requires the graphics to be upside down. 2 cycles doesn't sound like much, but in a scanline that's 2.6% of your processing time and saving it might be what allows you to update everything you want. In Kernels I've written, I often use every cycle—and that includes eliminating the sta WSYNC to buy back 3 cycles of processing time. See the reposition kernels in this post about Draconian.

I've also added joystick support that will let you move around the players. Pressing FIRE will slow down the movement, making it easier to line things up. The score (on the left) is used to display player0's Y position, and the timer is used for player1. As an added bonus, I'm showing how you can save ROM space by creating graphics that only face in one direction by using REFP0 and REFP1 (REFlect Player) to make the graphics face the other way. The routine's fairly sizable, so I'm not posting it here. Download the source code and check it out!

# Step 5: Automate Vertical Delay

For this update, we're going to double the Y range of the player objects. To use the new Y value for the 2LK we just need to divide it in half using the LSR command. The remainder of the divide, which ends up in the Carry flag, will conveniently tell us if we need to turn on Vertical Delay.

This routine preps the 2LK data for player0 and turns on VDELP0 if required (if you're wondering, VDELP0 is turned off in VerticalSync):

; prep Humanoid's Y position for 2LK

ldx #1              ; preload X for setting VDELPx

lda ObjectY         ; get the human's Y position

lsr                 ; divide by 2 for the 2LK position

sta Temp            ; save for position calculations

bcs NoDelay0        ; if carry is set we don't need Vertical Delay

stx VDELP0          ; carry was clear, so set Vertical Delay

NoDelay0:

; HumanDraw = ARENA_HEIGHT + HUMAN_HEIGHT - Y position

lda #(ARENA_HEIGHT + HUMAN_HEIGHT)

sec

sbc Temp

sta HumanDraw

; HumanPtr = HumanGfx + HUMAN_HEIGHT - 1 - Y position

lda #<(HumanGfx + HUMAN_HEIGHT - 1)

sec

sbc Temp

sta HumanPtr

lda #>(HumanGfx + HUMAN_HEIGHT - 1)

sbc #0

sta HumanPtr+1

One minor problem with the prior 2LK was that player1 could not show up on the topmost scanline of the Arena:

Closeup:

To fix this, we'll modify the kernel to prime GRP1 before it enters the loop that draws the Arena:

ldy #ARENA_HEIGHT+1 ; 2  7 - the arena will be 180 scanlines (from 0-89)*2

; prime GRP1 so player1 can appear on topmost scanline of the Arena

lda #BOX_HEIGHT-1   ; 2  9 - height of the box graphics,

dcp BoxDraw         ; 5 14 - Decrement BoxDraw and compare with height

bcs DoDrawGrp1pre   ; 2 16 - (3 17) if Carry is Set, then box is on current scanline

lda #0              ; 2 18 - otherwise use 0 to turn off player1

.byte $2C           ; 4 22 - $2C = BIT with absolute addressing, trick that

;        causes the lda (BoxPtr),y to be skipped

DoDrawGrp1pre:              ;   17 - from bcs DoDrawGRP1pre

lda (BoxPtr),y      ; 5 22 - load the shape for the box

sta GRP1            ; 3 25 - @0-22, update player1 to draw box

dey                 ; 2 27

ArenaLoop:                  ;   13 - from bpl ArenaLoop

The 2LK calculations for player1 used to be the same as for player0, but now must be modified to compensate for the priming of GRP1:

; prep box's Y position for 2LK

lda ObjectY+1       ; get the box's Y position

clc

adc #1              ; add 1 to compensate for priming of GRP1

lsr                 ; divide by 2 for the 2LK position

sta Temp            ; save for position calculations

bcs NoDelay1        ; if carry is set we don't need Vertical Delay

stx VDELP1          ; carry was clear, so set Vertical Delay

NoDelay1:

; BoxDraw = ARENA_HEIGHT + BOX_HEIGHT - Y position + 1

; the + 1 compensates for priming of GRP1

lda #(ARENA_HEIGHT + BOX_HEIGHT +1)

sec

sbc Temp

sta BoxDraw

; BoxPtr = BoxGfx + BOX_HEIGHT - 1 - Y position

lda #<(BoxGfx + BOX_HEIGHT - 1)

sec

sbc Temp

sta BoxPtr

lda #>(BoxGfx + BOX_HEIGHT - 1)

sbc #0

sta BoxPtr+1

Added GRP1 priming which allows player1 to cover full Arena:

Closeup:

Lastly, I added a new Box graphic for player1:

# Step 6: Spec Change

As anybody involved in writing software can tell you, project specifications will often change when new information becomes available.

When I started working on Collect, my plan was to use it for my presentation at Classic Game Fest. As I've progressed I've come to the realization that a full blown game is going to be just too much information for a one hour presentation. I decided that I'm going to leave the existing example in place and just add a few slides about Collect with a link to these blog entries for anybody who is interested.

Since I'm no longer planning to fit this project into a presentation, I've decided on a few changes:

- Make Collect a 1-2 player game

- Add the Ball object to draw an additional box, especially useful for 2 player games

- Change Timer display to the right player's score

- Add a Timer Bar to indicate time remaining

Mockup of two player game with new timer bar:

Timer has decreased:

One player variation will hide right player's score and use player1 as an additional box:

The Ball Object has a vertical delay feature. When used, the ball should be updated on the same scanline as player0. Due to this, I've revised the 2LK to be like this:

- update player1, update playfield, precalc player0 data for next scanline

- update player0, update playfield, precalc player1 data for next scanline

This was done to plan ahead for when the playfield is no longer updated on every scanline. Updating the playfield and ball will make the 2LK look something like this:

- update player1, update playfield, precalc player0 and ball data for next scanline

- update player0, update ball, precalc player 1 data for next scanline

I'll also need to add in updates for the missiles. Ideally we want to update them on every scanline like this:

- update player1, update missile0, update missile1, update playfield, precalc player0 and ball and missile data for next scanline

- update player0, update missile0, update missile1, update ball, precalc player 1 and missile data for next scanline

It's possible the timing won't work out for that. If it doesn't, then a change like this should work:

- update player1, update missile1, update playfield, precalc player0 and ball and missile0 data for next scanline

- update player0, update missile0, update ball, precalc player1 and missile1 data for next scanline

That would make it so that the missile objects can only start on every-other-scanline, but that's an OK compromise for our game.

In this build I've revised the Arena to be a little bit shorter to make room for the new timer display. The timer currently "ticks" once every 64 frames. Whenever it ticks, a bunch of byte rotations are done to shorten the length of the timer bar.

DecrementTimer:

lsr TimerPF+5   ; PF2 right side, reversed bits so shift right

rol TimerPF+4   ; PF1 right side, normal bits so shift left

ror TimerPF+3   ; PF0 right side, reversed bits so shift right

lda TimerPF+3   ; only upper nybble used, so we need to put bit 3 into C

lsr

lsr

lsr

lsr

ror TimerPF+2   ; PF2 left side, reversed bits so shift right

rol TimerPF+1   ; PF1 left side, normal bits so shift left

ror TimerPF     ; PF0 left side, reversed bits so shift right

rts

Since there are 40 playfield pixels, the total playtime would be 40*64/60 = 42.7 seconds. We might decide that's too short of a play time. If so, we'll just change the tick to occur every 128 frames for 40*128/60 = 85.3 seconds of game time, or maybe even once very 256 frames for 40*256/60 = 170.7 seconds.

SetObjectColors has been modified to add a color for the timer bar. The Timer Bar and the Arena are both drawn using the playfield, so to make the Arena a different color than the Timer Bar I store the current Arena color in a RAM location.

SetObjectColors:

ldx #4          ; we're going to set 5 colors (0-4)

ldy #4          ; default to the color entries in the table (0-4)

lda SWCHB       ; read the state of the console switches

and #%00001000  ; test state of D3, the TV Type switch

bne SOCloop     ; if D3=1 then use color

ldy #9          ; else use the b&w entries in the table (5-9)

SOCloop:

lda Colors,y    ; get the color or b&w value

sta COLUP0-1,x  ; and set it

dey             ; decrease Y

dex             ; decrease X

bne SOCloop     ; Branch Not Equal to Zero

lda Colors,y    ; get the Arena color

sta ArenaColor  ; save in RAM for Kernal Usage

rts             ; ReTurn from Subroutine

Colors:

.byte $46   ; red        - goes into COLUPF, color for Arena (after Timer is drawn)

.byte $86   ; blue       - goes into COLUP0, color for player0 and missile0

.byte $C6   ; green      - goes into COLUP1, color for player1 and missile1

.byte $64   ; purple     - goes into COLUPF, color for Timer

.byte $00   ; black      - goes into COLUBK, color for background

.byte $0A   ; light grey - goes into COLUPF, color for Arena (after Timer is drawn)

.byte $0E   ; white      - goes into COLUP0, color for player0 and missile0

.byte $06   ; dark grey  - goes into COLUP1, color for player1 and missile1

.byte $04   ; dark grey  - goes into COLUPF, color for Timer

.byte $00   ; black      - goes into COLUBK, color for background

For testing, I've set it up so the Right Difficulty switch is used to determine if the game is a one or two player game for which graphics to use for player1:

ldx #0

bit SWCHB

bpl TwoPlayer

ldx #1

TwoPlayer:

; Player1Ptr = BoxGfx + HUMAN_HEIGHT - 1 - Y position

lda ShapePtrLow,x

sec

sbc Temp

sta Player1Ptr

lda ShapePtrHi,x

sbc #0

sta Player1Ptr+1

rts

ShapePtrLow:

.byte <(HumanGfx + HUMAN_HEIGHT - 1)

.byte <(BoxGfx + HUMAN_HEIGHT - 1)

ShapePtrHi:

.byte >(HumanGfx + HUMAN_HEIGHT - 1)

.byte >(BoxGfx + HUMAN_HEIGHT - 1)

Right Difficulty = B:

Right Difficulty = A:

# Step 7: Draw the Playfield

For this update, we're going to modify the Arena Loop to draw the Arena using the playfield. The new Arena loop has these new changes:

ArenaLoop:                  ;   27 - (currently 7 from bpl ArenaLoop)

tya                 ; 2 29 - 2LK loop counter in A for testing

and #%11            ; 2 31 - test for every 4th time through the loop,

bne SkipX           ; 2 33 (3 34) branch if not 4th time

inx                 ; 2 35 - if 4th time, increase X so new playfield data is used

SkipX:                      ;   35 - use 35 as it's the longest path here

...

; start of line 1 of the 2LK

sta GRP1            ; 3  3 - @0-22, update player1 graphics

lda ArenaPF0,x      ; 4  7 - get current scanline's playfield pattern

sta PF0             ; 3 10 - @0-22 and update it

lda ArenaPF1,x      ; 4 14 - get current scanline's playfield pattern

sta PF1             ; 3 17 - @71-28 and update it

lda ArenaPF2,x      ; 4 21 - get current scanline's playfield pattern

sta PF2             ; 3 24 - @60-39

...

; start of line 2 of the 2LK

sta GRP0            ; 3  3 - @0-22, update player0 graphics

dey                 ; 2  5 - decrease the 2LK loop counter

bne ArenaLoop       ; 2  7 - (3  8) branch if there's more Arena to draw

sty PF0             ; 3 10 - Y is 0, blank out playfield

sty PF1             ; 3 13 - Y is 0, blank out playfield

sty PF2             ; 3 16 - Y is 0, blank out playfield

rts                 ; 6 22 - ReTurn from Subroutine

The first change is we're using X as an index into the playfield graphic data. We're changing X every fourth time thru the 2LK, so each byte of playfield data will be used over 8 scanlines. This saves a bit of ROM.

Second change is all 3 playfield registers (PF0, PF1 and PF2) are now updated, and they're only updated on line 1 of our 2LK.

Third change is on line 2, the bpl ArenaLoop is now a bne ArenaLoop else the bottom row of playfield data was only used for 2 scanlines instead of 8. We also blank out the playfield registers when we are done drawing the playfield. The bne change also impacted Overscan—TIM64T was originally set to 32, it's now set to 35.

The playfield data looks like this in jEdit:

And this onscreen:

Lastly we added some collision detection code. Some space was allocated in RAM:

;save player locations for playfield collision logic

SavedX:         ds 2    ; stored in $A1-A2

SavedY:         ds 2    ; stored in $A3-A4

Then the Process Joystick routines save the current X and Y values before processing the joystick:

PJloop:

ldy ObjectX,x   ; save original X location so the player can be

sty SavedX,x    ;   bounced back upon colliding with the playfield

ldy ObjectY,x   ; save original Y location so the player can be

sty SavedY,x    ;   bounced back upon colliding with the playfield

Finally OverScan was modified to move the players back to their previous X and Y location if a collision was detected:

; Test if player collided with playfield

bit CXP0FB      ; N = player0/playfield, V=player0/ball

bpl notP0PF     ; if N is off, then player0 did not collide with playfield

lda SavedX      ; recall saved X

sta ObjectX     ; and move player back to it

lda SavedY      ; recall saved Y

sta ObjectY     ; and move player back to it

notP0PF:

bit CXP1FB      ; N = player1/playfield, V=player1/ball

bpl notP1PF     ; if N is off, then player1 did not collide with playfield

lda SavedX+1    ; recall saved X

sta ObjectX+1   ; and move player back to it

lda SavedY+1    ; recall saved Y

sta ObjectY+1   ; and move player back to it

notP1PF:

# Step 8: Select and Reset Support

For this update we're adding initial support for the Select and Reset buttons. For this we're adding a new RAM variable called GameState to keep track of "Game Active" vs "Game Over".

; D7, 1=Game Active, 0=Game Over

GameState:      ds 1    ; stored in $A7

We're going to use D7 to denote the state as we can easily test D7 (as well as D6) by using the BIT command. You can see this in the revised Vertical Blank routine were we test GameState to determine if UpdateTimer and ProcessJoystick should be skipped over:

VerticalBlank:

jsr ProcessSwitches

bit GameState

bpl NotActive

jsr UpdateTimer

jsr ProcessJoystick

NotActive:

jsr PositionObjects

jsr SetObjectColors

jsr PrepScoreForDisplay

rts             ; ReTurn from Subroutine

ProcessSwitches will check SWCHB to see if RESET is pressed. If so, it'll start up a new game. If not, it'll check if SELECT is pressed, and if so cancel an active game.

ProcessSwitches:

lda SWCHB       ; load in the state of the switches

lsr             ; D0 is now in C

bcs NotReset    ; if D0 was on, the RESET switch was not held

jsr InitPos     ; Prep for new game

lda #%10000000

sta GameState   ; set D7 on to signify Game Active

rts

NotReset:

lsr             ; D1 is now in C

bcs NotSelect

lda #0

sta GameState   ; clear D7 to signify Game Over

NotSelect:

rts

In the next update ProcessSwitches will be expanded upon so that the Select routine will let you select a game variation (and if you check the source you'll see a new Arena layout is already in place for that).

In order to visually show you that the game is over, I've revised the Color routines to color cycle if the game is not active.

SetObjectColors:

lda #$FF

sta Temp2       ; default to color mask

and ColorCycle  ; color cycle

bit GameState

bpl SOCgameover

lda #0          ; if game is active, no color cycle

SOCgameover:

sta Temp

ldx #4          ; we're going to set 5 colors (0-4)

ldy #4          ; default to the color entries in the table (0-4)

lda SWCHB       ; read the state of the console switches

and #%00001000  ; test state of D3, the TV Type switch

bne SOCloop     ; if D3=1 then use color

ldy #$0f

sty Temp2       ; set B&W mask

ldy #9          ; else use the b&w entries in the table (5-9)

SOCloop:

lda Colors,y    ; get the color or b&w value

eor Temp        ; color cycle

and Temp2       ; B&W mask

sta COLUP0-1,x  ; and set it

dey             ; decrease Y

dex             ; decrease X

bne SOCloop     ; Branch Not Equal to Zero

lda Colors,y    ; get the Arena color

eor Temp        ; color cycle

and Temp2       ; B&W mask

sta ArenaColor  ; save in RAM for Kernal Usage

rts             ; ReTurn from Subroutine

Color cycle example:

B&W Color Cycle example:

# Step 9: Game Variations

It's common for Atari games to have a number of game variations. To simplify the logic, the variations are usually driven by individual bits and/or groups of bits within a single byte that holds the game variation. A good example of that would be Space Invaders—check out the game matrix from the manual:

A byte is comprised of 8 bits, usually numbered 0-7 where 7 is the leftmost bit as in 76543210.

For Space Invaders, the bits in the game variation are used in this fashion:

- 7 - not used

- 654 - selects 1 of 7 player variations

- Invisible Invaders

- Fast Bombs

- ZigZagging Bombs

- Moving Shields

Humans start counting at 1, but computers start at 0, so if you select game variation 12, internally it's really 11. 11 in binary is %00001011, which means bits 0, 1 and 3 are all turned on so the game variation has Invisible Invaders, ZigZagging Bombs and Moving Shields. You can confirm that by looking at column 12 of the Game Matrix above.

With this update to Collect, we're using bits 1 and 0 to give us 4 game variations:

- 765432 - not used

- 1 - Arena 1 or 2

- 0 - # of players

If we have space at the end of the project, I plan to add some additional arenas. If we can add 2 more we'd just start using bit 2 and let the game Variation go from 1-8:

- 76543 - not used

- 21 - Arena 1, 2, 3 or 4

- 0 - # of players

If we can add 6 more we'd add bit 3 and let the game variation go from 1-16

- 76543 - not used

- 321 - Arena 1, 2, 3, 4, 5, 6, 7 or 8

- 0 - # of players

The ProcessSwitches routine has been modified so that hitting Select will increment the new variable Variation. It will also limit Variation to only the values 0-3. After changing Variation, the left score will be set to show Variation+1 as Humans prefer to see 1-4 instead of 0-3. The Right Score will be used to show the # of players, either 1 or 2.

NotReset:

lsr             ; D1 is now in C

bcs NotSelect   ; if D1 was on, the SELECT switch was not held

lda #0

sta GameState   ; clear D7 to signify Game Over

ldx Variation   ; Get the Game Variation

inx             ; and increase it

txa             ; transfer it to A

and #%00000011  ; limit Variation to 0-3

sta Variation   ; save it

tax             ; transfer it to X

inx             ; and increase it by 1 for the human readable varation 1-4

stx Score       ; save in Score so it shows on left side

ldy #1          ; default to showing 1 player variation

lsr             ; D0 of Variation, # of players, now in Carry flag

bcc Not2        ; if Carry is clear, then show 1 player

iny             ; else set Y to 2 to show 2 players

Not2:

ror Players     ; put Carry into D7 for BIT testing of # of players

sty Score+1     ; show the human readable # of players on right side

NotSelect:

rts

The routine works, but when Select is pressed the game variation will rapidly change making it difficult to select a specific game variation. You can see that in the build in my blog entry.

To fix that, we'll add a SelectDelay variable so that holding down SELECT will only result in Variation changing at the rate of once per second. However, if the user rapidly presses/releases SELECT then Variation will also rapidly change.

ProcessSwitches:

lda SWCHB       ; load in the state of the switches

lsr             ; D0 is now in C

bcs NotReset    ; if D0 was on, the RESET switch was not held

jsr InitPos     ; Prep for new game

lda #%10000000

sta GameState   ; set D7 on to signify Game Active

bne NotSelect   ; clear SelectDelay

NotReset:

lsr             ; D1 is now in C

bcs NotSelect   ; if D1 was on, the SELECT switch was not held

lda #0

sta GameState   ; clear D7 to signify Game Over

lda SelectDelay ; do we need to delay the Select switch?

beq SelectOK    ; if delay is 0 then no

dec SelectDelay ; else decrement the delay

rts             ; and exit the subroutine

SelectOK:

lda #60         ; Set the Select Delay to 1 second

sta SelectDelay ;

ldx Variation   ; Get the Game Variation

inx             ; and increase it

txa             ; transfer it to A

and #%00000011  ; limit Variation to 0-3

sta Variation   ; save it

tax             ; transfer it to X

inx             ; and increase it by 1 for the human readable varation 1-4

stx Score       ; save in Score so it shows on left side

ldy #1          ; default to showing 1 player variation

lsr             ; D0 of Variation, # of players, now in Carry flag

bcc Not2        ; if Carry is clear, then show 1 player

iny             ; else set Y to 2 to show 2 players

Not2:

ror Players     ; put Carry into D7 for BIT testing of # of players

sty Score+1     ; show the human readable # of players on right side

rts

NotSelect:

lda #0          ; clears SelectDelay if SELECT not held

sta SelectDelay

rts

The routine PositionObjects has been modified to use a Box Graphic for player1 if a 1 player game has been selected:

PositionObjects:

...

lda Variation       ; get the game variation

and #1              ; and find out if we're 1 or 2 player

tax

; Player1Ptr = BoxGfx + HUMAN_HEIGHT - 1 - Y position

lda ShapePtrLow,x

sec

sbc Temp

sta Player1Ptr

lda ShapePtrHi,x

sbc #0

sta Player1Ptr+1

rts

ShapePtrLow:

.byte <(BoxGfx + HUMAN_HEIGHT - 1)

.byte <(HumanGfx + HUMAN_HEIGHT - 1)

ShapePtrHi:

.byte >(BoxGfx + HUMAN_HEIGHT - 1)

.byte >(HumanGfx + HUMAN_HEIGHT - 1)

The Kernel has also been modified so that the correct Arena will be drawn. A little bit after TimerBar: you'll find this:

TimerBar:

...

lda Variation       ; 3 20

lsr                 ; 2 22 - which Arena to show

tay                 ; 2 24 - set for index

ldx ArenaOffset,y   ; 4 28 - set X for which arena to draw

lda ArenaPF0,x      ; 4 32 - reflect and priority for playfield

and #%00000111      ; 2 34 - get the lower 3 bits for CTRLPF

ora #%00110000      ; 2 36 - set ball to display as 8x pixel

sta CTRLPF          ; 3 39

...

ArenaOffset:

.byte 0         ; Arena 1

.byte 22        ; Arena 2

The lsr command shifts bit 1 down to bit 0 so that we end up with 0 or 1 for the Arena number. That's used to set X to either 0 or 22 via the command ldx ArenaOffset,y.

I also added code to update CTRLPF based on the first PF0 data byte for the selected Arena. CTRLPF uses its bits like this:

- 76 - not used

- 54 - set width of BALL object

- 3 - not used

- 2 - Playfield Priority

- 1 - Score Mode

- 0 - Reflected Playfield

Since PF0 only uses bits 7654, also known as the upper nybble of the byte, we can use the lower nybble to hold extra information to specify whether or not the selected Arena uses Playfield Priority (as opposed to Player Priority) or has a Reflected Playfield(as opposed to Repeated Playfield). We could even specify Score Mode which would just color the two sides of the playfield to match the colors of the players (like in the score display).

ArenaPF0:   ; PF0 is drawn in reverse order, and only the upper nybble

.byte %11110001 ; Arena 1   lower nybble controls playfield, set for REFLECT

.byte %00010000

.byte %00010000

.byte %00010000

...

.byte %11110100 ; Arena 2 - lower nybble controls playfield, set for PLAYFIELD PRIORITY

.byte %00010000

.byte %00010000

.byte %00010000

Game Variation 2, Arena 1 with 2 players. Arena 1 features Reflected Playfield and Player Priority:

Game

Variation 3, Arena 2 with 1 player. Arena 2 features Repeated Playfield and Playfield Priority:

Look at the left Humanoid's head in each screenshot to see the difference that setting Playfield Priority makes. You might remember this being used in some games like Combat where the planes go behind the clouds.

Just for fun, here's Arena 2 with SCORE mode set (I've moved the players to the side of the screen they didn't start on):

The code change for that is:

.byte %11110010 ; Arena 2 - lower nybble controls playfield, set for SCORE

# Step 10: “Random” Numbers

And yes, those are air quotes. Wink

Inside the Atari (and computers in general), there's no such thing as a Random Number. We can, however, simulate random numbers by using a Linear Feedback Shift Register. The LFSR we're going to use was posted by batari, it's a rather slick bit of code in that it can be either an 8-bit or 16-bit LFSR.

Random:

lda Rand8

lsr

ifconst Rand16

rol Rand16      ; this command is only used if Rand16 has been defined

endif

bcc noeor

eor #$B4

noeor

sta Rand8

ifconst Rand16

eor Rand16      ; this command is only used if Rand16 has been defined

endif

rts

LFSR's create what appear to be a random sequence of numbers, but they're not. A properly constructed 8-bit LFSR will start repeating values after 255 values have been obtained. All you need to do in order to use the above routine as an 8-bit LFSR is allocate a variable called Rand8:

; used by Random for an 8 bit random number

Rand8:          ds 1    ; stored in $AB

A 16-bit LFSR will repeat after 65535 values have been obtained—that's a lot better than 255; however, it requires you to allocate another variable. With the Atari's limited 128 bytes of RAM, games often didn't have RAM to spare so they'd just use an 8-bit LFSR. Collect is a rather simple game though, so we have RAM to spare and will allocate the second variable Rand16.

; optionally define space for Rand16 for 16 bit random number

Rand16:         ds 1    ; stored in $AC

The random number generator does have potential problem to be aware of—if you're using an 8-bit LFSR and Rand8 has the value of 0 then the LFSR will always return 0. Likewise for the 16-bit LFSR if both Rand8 and Rand16 are 0 it will always return 0. So we need to initialize the LFSR as CLEAN_START set all the RAM variables to 0.

InitSystem:

CLEAN_START

...

; seed the random number generator

lda INTIM       ; unknown value

sta Rand8       ; use as seed

eor #$FF        ; both seed values cannot be 0, so flip the bits

sta Rand16      ;   just in case INTIM was 0

One thing that helps when using an LFSR is to keep reading values at a regular rate, even if you don't need the value. What this does is impose an element outside of the Atari's control—namely the time it takes the human to things: hit the RESET switch to start a game, collect the next box, etc. I've added this to VerticalBlank:

VerticalBlank:

jsr Random

...

Now that we have a function for random numbers we need to use them. First up is new routine that will randomly position any object. To use this function we just need to call it with X register holding a value that denotes which object to position. The values are the same ones used for the PosObject function, namely 0=player0, 1=player1, 2=missile0, 3=missile1 and 4=ball.

RandomLocation:

jsr Random      ; get a random value between 0-255

and #127        ; limit range to 0-127

sta Temp        ; save it

jsr Random      ; get a random value between 0-255

and #15         ; limit range to 0-15

clc             ; must clear carry for add

adc Temp        ; add in random # from 0-127 for range of 0-142

adc #5          ; add 5 for range of 5-147

sta ObjectX,x   ; save the random X position

jsr Random      ; get a random value between 0-255

and #127        ; limit range to 0-127

sta Temp        ; save it

jsr Random      ; get a random value between 0-255

and #15         ; limit range to 0-15

clc             ; must clear carry for add

adc Temp        ; add in random # from 0-127 for range of 0-142

adc #26         ; add 26 for range of 26-168

sta ObjectY,x   ; save the random Y position

rts

I've renamed InitPos to NewGame and modified it to call RandomLocation. It runs through a loop setting all the box objects. Starting X value will be 1 or 2 based on the number of players in the selected game variation.

NewGame:

...

; Randomly position the boxes for the new game.  Set X to 1 for a 1 player

; game or 2 for a 2 player game so that the appropriate objects will be

; randomly placed in the Arena.

lda Variation

and #1              ; value of 0=1 player game, 1=2 player game

tax                 ; transfer to X

inx                 ; start with 1 for a 1 player game, or 2 for a 2 player game

IPloop:

jsr RandomLocation  ; randomly position object specified by X

inx                 ; increase X for next object

cpx #5              ; check if we hit 5

bne IPloop          ; branch back if we haven't

...

rts

I also modified OverScan so that during a 1-player variation a collision between player0 and player1 will be detected. If so, it calls another new function, CollectBox.

OverScan:

...

bit Players     ; test how many players are in this game variation

bmi RightPlayer ; test Right Player collisions if its a 2 player game

bit CXPPMM      ; else see if left player collected box drawn by player1

bpl OSwait      ; player0 did not collide wth player1

ldx #1          ; which box was collected

jsr CollectBox  ; update score and reposition box

jmp OSwait      ; 1 player game, so skip Right Player test

CollectBox will increase the player's score and call RandomLocation again to move it to a new position. When CollectBox is called, Y must hold which player (0 for left, 1 for right) and X must hold the object that was collected.

CollectBox:

SED                 ; SEt Decimal flag

clc                 ; CLear Carry bit

lda #1              ; 1 point per box

adc Score,y         ; add to player's current score

sta Score,y         ; and save it

CLD                 ; CLear Decimal flag

jsr RandomLocation  ; move box to new location

rts

CollectBox means that the game is now playable for 1-player games! I managed to score 26 on game variation 1, how well can you do?

One thing you might notice in that screenshot is that the right player's score is not visible. Since we're done using the score for diagnostics, I've made a few changes to the digit graphics:

The first change is the left 0 image was blanked out—this provides leading zero suppression:

The second change is the A image is now blanked out—NewGame uses this to blank out the right player's score in 1 player games.

NewGame:

...

; reset scores

ldx #0

stx Score

bit Players         ; check # of players

bpl BlankRightScore

stx Score+1

rts

BlankRightScore:

lda #$AA            ; AA defines a "space" character

sta Score+1

rts

Lastly, the graphics for B through F have been removed to save ROM space.

# Step 11: Add the Ball Object

The 2LK has been revised to display the ball object. Like player0, the ball needs to be primed before the ArenaLoop begins:

; prime ENABL so ball can appear on topmost scanline of Arena

ldx #1              ; 2  2 - D1=0, so ball will be off

lda #BOX_HEIGHT-1   ; 2  4 - height of box graphic

dcp BallDraw        ; 5  9 - Decrement BallDraw and compare with height

bcs DoEnablPre      ; 2 11 - (3 12) if Carry is Set, then ball is on current scanline

.byte $24           ; 3 14 - $24 = BIT with zero page addressing, trick that

;        causes the inx to be skipped

DoEnablPre:                 ;   12 - from bcs DoEnablPre

inx                 ; 2 14 - D1=1, so ball will be ON

stx ENABL           ; 3 17

; prime GRP0 so player0 can appear on topmost scanline of the Arena

lda #HUMAN_HEIGHT-1 ; 2 19 - height of player0 graphics,

dcp Player0Draw     ; 5 24 - Decrement Player0Draw and compare with height

bcs DoDrawGrp0pre   ; 2 26 - (3 27) if Carry is Set, then player0 is on current scanline

lda #0              ; 2 28 - otherwise use 0 to turn off player0

.byte $2C           ; 4 32 - $2C = BIT with absolute addressing, trick that

;        causes the lda (Player0Ptr),y to be skipped

DoDrawGrp0pre:              ;   27 - from bcs DoDrawGRP0pre

lda (Player0Ptr),y  ; 5 32 - load the shape for player0

sta GRP0            ; 3 35 - @0-22, update player0 graphics

dey                 ; 2 37

ArenaLoop:                  ;   37 - (currently 11 from bpl ArenaLoop)

tya                 ; 2 39 - 2LK loop counter in A for testing

and #%11            ; 2 41 - test for every 4th time through the loop,

bne SkipX           ; 2 43 - (3 44) branch if not 4th time

inc ArenaIndex      ; 5 48 - if 4th time, increase index so new playfield data is used

SkipX:                      ;   48 - use 48 as it's the longest path here

; continuation of line 2 of the 2LK

; this precalculates data that's used on line 1 of the 2LK

lda #HUMAN_HEIGHT-1 ; 2 50 - height of the humanoid graphics, subtract 1 due to starting with 0

dcp Player1Draw     ; 5 55 - Decrement Player1Draw and compare with height

bcs DoDrawGrp1      ; 2 57 - (3 58) if Carry is Set, then player1 is on current scanline

lda #0              ; 2 59 - otherwise use 0 to turn off player1

.byte $2C           ; 4 63 - $2C = BIT with absolute addressing, trick that

;        causes the lda (Player1Ptr),y to be skipped

DoDrawGrp1:                 ;   58 - from bcs DoDrawGrp1

lda (Player1Ptr),y  ; 5 63 - load the shape for player1

sta WSYNC           ; 3 66

;---------------------------------------

; start of line 1 of the 2LK

sta GRP1            ; 3  3 - @0-22, update player1 graphics

ldx ArenaIndex      ; 3  6

lda ArenaPF0,x      ; 4 10 - get current scanline's playfield pattern

sta PF0             ; 3 13 - @0-22 and update it

lda ArenaPF1,x      ; 4 17 - get current scanline's playfield pattern

sta PF1             ; 3 20 - @71-28 and update it

lda ArenaPF2,x      ; 4 24 - get current scanline's playfield pattern

sta PF2             ; 3 27 - @60-39

; precalculate data that's needed for line 2 of the 2LK

ldx #1              ; 2 29 - D1=0, so ball will be off

lda #BOX_HEIGHT-1   ; 2 31 - height of box graphic

dcp BallDraw        ; 5 36 - Decrement BallDraw and compare with height

bcs DoEnabl         ; 2 38 - (3 39) if Carry is Set, then ball is on current scanline

.byte $24           ; 3 41 - $24 = BIT with zero page addressing, trick that

;        causes the inx to be skipped

DoEnabl:                    ;   39 - from bcs DoEnablPre

inx                 ; 2 41 - D1=1, so ball will be ON

lda #HUMAN_HEIGHT-1 ; 2 43 - height of the box graphics,

dcp Player0Draw     ; 5 48 - Decrement Player0Draw and compare with height

bcs DoDrawGrp0      ; 2 50 - (3 51) if Carry is Set then player0 is on current scanline

lda #0              ; 2 52 - otherwise use 0 to turn off player0

.byte $2C           ; 4 56 - $2C = BIT with absolute addressing, trick that

;        causes the lda (Player0Ptr),y to be skipped

DoDrawGrp0:                 ;   51 - from bcs DoDrawGRP0

lda (Player0Ptr),y  ; 5 56 - load the shape for player0

sta WSYNC           ; 3 59

;---------------------------------------

; start of line 2 of the 2LK

sta GRP0            ; 3  3 - @0-22, update player0 graphics

stx ENABL           ; 3  6 - @0-22, update ball graphics

dey                 ; 2  8 - decrease the 2LK loop counter

bne ArenaLoop       ; 2 10 - (3 11) branch if there's more Arena to draw

sty PF0             ; 3 13 - Y is 0, blank out playfield

sty PF1             ; 3 16 - Y is 0, blank out playfield

sty PF2             ; 3 19 - Y is 0, blank out playfield

rts                 ; 6 25 - ReTurn from Subroutine

I then modified RandomLocation to set all objects to the same location for comparison:

RandomLocation:

...

; for alignment test, set to (100, 100)

lda #100

sta ObjectX,x

sta ObjectY,x

rts

This revealed a minor quirk with the TIA—namely that when objects are set to the same X position, missiles and the ball end up 1 pixel to the left of where a player ends up (player1 is the green square and it's directly on top of the red ball).

This is a known issue and the solution is to increase the X value by 1 to compensate. I did so by adding this to the end of RandomLocation:

RandomLocation:

...

cpx #2

bcc RLdone

inc ObjectX,x   ; missile and ball objects need their X adjusted

RLdone:

rts

And now player1 and the ball line up:

The NewGame routine revised last time already sets the ball to a random X-Y location, so all that's left to make it show up is to revise PositionObjects. Two changes are needed, first is to set the X position of the ball object by starting the POloop with X=4 (in the prior build X was initialized to 1), then prep a new variable BallDraw that's used by the 2LK to draw the ball object on the correct scanlines.

PositionObjects:

ldx #4              ; position all objects

POloop

lda ObjectX,x       ; get the object's X position

jsr PosObject       ; set coarse X position and fine-tune amount

dex                 ; DEcrement X

bpl POloop          ; Branch PLus so we position all objects

sta WSYNC           ; wait for end of scanline

sta HMOVE           ; use fine-tune values to set final X positions

...

; prep ball's Y position for 2LK

ldx #1              ; preload X for setting VDELBL

lda ObjectY+4       ; get the balls's Y position

clc

adc #1              ; add 1 to compensate for priming of ball

lsr                 ; divide by 2 for the 2LK position

sta Temp            ; save for position calculations

bcs NoDelayBL        ; if carry is set we don't need Vertical Delay

stx VDELBL          ; carry was clear, so set Vertical Delay

NoDelayBL:

; BallDraw = ARENA_HEIGHT + BOX_HEIGHT - Y position + 1

; the + 1 compensates for priming of ENABL

lda #(ARENA_HEIGHT + BOX_HEIGHT + 1)

sec

sbc Temp

sta BallDraw

rts

Lastly, Overscan's been updated to process collisions with the ball.

OverScan:

...

TestCollisions:

; Test left player collisions

...

bit CXP0FB      ; N = player0/playfield, V=player0/ball

...

notP0PF:                ; oVerflow flag is not affected by lda or sta

bvc notP0BL     ; if V is off, then player0 did not collide with ball

ldx #4          ; which box was collected

jsr CollectBox  ; update score and reposition box

notP0BL:

...

RightPlayer:

; Test right player collisions

...

bit CXP1FB      ; N = player1/playfield, V=player1/ball

...

notP1PF:                ; oVerflow flag is not affected by lda or sta

bvc notP1BL     ; if V is off, then player1 did not collide with ball

ldx #4          ; which box was collected

jsr CollectBox  ; update score and reposition box

notP1BL:

2 player games are now playable:

And 1 player games have 2 boxes to collect:

# Step 12: Add the Missile Objects

The 2LK has been revised to display both missile objects. Missile objects do not support Vertical Delay, so there's no need to prime ENAM0 or ENAM1.

ArenaLoop:                  ;   17 - (currently 17 from bpl ArenaLoop)

; continuation of line 2 of the 2LK

; this precalculates data that's used on line 1 of the 2LK

tya                 ; 2 19 - 2LK loop counter in A for testing

and #%11            ; 2 21 - test for every 4th time through the loop,

bne SkipX           ; 2 23 - (3 24) branch if not 4th time

inc ArenaIndex      ; 5 28 - if 4th time, increase index so new playfield data is used

SkipX:                      ;   28 - use 28 as it's the longest path to here

ldx #1              ; 2 30 - D1=0, so missile1 will be off

lda #BOX_HEIGHT-1   ; 2 32 - height of box graphic

dcp Missile1Draw    ; 5 37 - Decrement Missile1Draw and compare with height

bcs DoEnam1         ; 2 39 - (3 40) if Carry is Set, then missile1 is on current scanline

.byte $24           ; 3 42 - $24 = BIT with zero page addressing, trick that

;        causes the inx to be skipped

DoEnam1:                    ;   40 - from bcs DoEnam1

inx                 ; 2 42 - D1=1, so ball will be ON

lda #HUMAN_HEIGHT-1 ; 2 44 - height of the humanoid graphics, subtract 1 due to starting with 0

dcp Player1Draw     ; 5 49 - Decrement Player1Draw and compare with height

bcs DoDrawGrp1      ; 2 51 - (3 52) if Carry is Set, then player1 is on current scanline

lda #0              ; 2 53 - otherwise use 0 to turn off player1

.byte $2C           ; 4 57 - $2C = BIT with absolute addressing, trick that

;        causes the lda (Player1Ptr),y to be skipped

DoDrawGrp1:                 ;   52 - from bcs DoDrawGrp1

lda (Player1Ptr),y  ; 5 57 - load the shape for player1

sta WSYNC           ; 3 60

;---------------------------------------

; start of line 1 of the 2LK

sta GRP1            ; 3  3 - @0-22, update player1 graphics

stx ENAM1           ; 3  6 - @0-22, update missile1 graphics

ldx ArenaIndex      ; 3  9

lda ArenaPF0,x      ; 4 13 - get current scanline's playfield pattern

sta PF0             ; 3 16 - @0-22 and update it

lda ArenaPF1,x      ; 4 20 - get current scanline's playfield pattern

sta PF1             ; 3 23 - @71-28 and update it

lda ArenaPF2,x      ; 4 27 - get current scanline's playfield pattern

sta PF2             ; 3 30 - @60-39

; precalculate data that's needed for line 2 of the 2LK

ldx #1              ; 2 32 - D1=0, so missile0 will be off

lda #BOX_HEIGHT-1   ; 2 34 - height of box graphic

dcp Missile0Draw    ; 5 39 - Decrement Missile0Draw and compare with height

bcs DoEnam0         ; 2 41 - (3 42) if Carry is Set, then missile0 is on current scanline

.byte $24           ; 3 44 - $24 = BIT with zero page addressing, trick that

;        causes the inx to be skipped

DoEnam0:                    ;   42 - from bcs DoEnam0

inx                 ; 2 44 - D1=1, so ball will be ON

stx Temp            ; 3 47 - save for line 2

ldx #1              ; 2 49 - D1=0, so ball will be off

lda #BOX_HEIGHT-1   ; 2 51 - height of box graphic

dcp BallDraw        ; 5 56 - Decrement BallDraw and compare with height

bcs DoEnabl         ; 2 58 - (3 59) if Carry is Set, then ball is on current scanline

.byte $24           ; 3 61 - $24 = BIT with zero page addressing, trick that

;        causes the inx to be skipped

DoEnabl:                    ;   59 - from bcs DoEnablPre

inx                 ; 2 61 - D1=1, so ball will be ON

lda #HUMAN_HEIGHT-1 ; 2 63 - height of the box graphics,

dcp Player0Draw     ; 5 68 - Decrement Player0Draw and compare with height

bcs DoDrawGrp0      ; 2 70 - (3 71) if Carry is Set then player0 is on current scanline

lda #0              ; 2 72 - otherwise use 0 to turn off player0

.byte $2C           ; 4 76 - $2C = BIT with absolute addressing, trick that

;        causes the lda (Player0Ptr),y to be skipped

; start of line 2 of the 2LK

DoDrawGrp0:                 ;   71 - from bcs DoDrawGRP0

lda (Player0Ptr),y  ; 5 76 - load the shape for player0

;---------------------------------------

; start of line 2 of the 2LK

sta GRP0            ; 3  3 - @0-22, update player0 graphics

stx ENABL           ; 3  6 - @0-22, update ball graphics

lda Temp            ; 3  9 - get the precalced data for missile0

sta ENAM0           ; 3 12 - @0-22, update missile0 graphics

dey                 ; 2 14 - decrease the 2LK loop counter

bne ArenaLoop       ; 2 16 - (3 17) branch if there's more Arena to draw

sty PF0             ; 3 19 - @0-22, Y is 0, blank out playfield

sty PF1             ; 3 22 - @71-28, Y is 0, blank out playfield

sty PF2             ; 3 25 - @60-39, Y is 0, blank out playfield

rts                 ; 6 31 - ReTurn from Subroutine

The 2 lines of the 2LK now work like this:

- updates player1, missile1, playfield, precalcs player0, missile0, ball for line 2

- updates player0, missile0, ball, precalcs player1, missile1 and playfield index for line 1

We didn't have enough time to update missile0 and missile1 on both lines of the 2LK. Since the TIA doesn't offer a vertical delay feature for the missiles, they can never line up. This is just like how the players didn't line up when VDELP0 and VDELP1 were not used. For our game, this isn't a problem.

One thing to notice about the revised 2LK is the 1st line uses exactly 76 cycles, so it no longer ends with a sta WSYNC.

PositionObjects has been modified to initialize 2 new variables, Missile0Draw and Missile1Draw, that are used by the 2LK to control when the missiles are drawn.

PositionObjects:

...

; prep missile0's Y position for 2LK

lda ObjectY+2       ; get the missile's Y position

lsr                 ; divide by 2 for 2LK

sta Temp            ; save for position calculation

; Missile0Draw = ARENA_HEIGHT + BOX_HEIGHT - Y position

lda #(ARENA_HEIGHT + BOX_HEIGHT)

sec

sbc Temp

sta Missile0Draw

; prep missile1's Y position for 2LK

lda ObjectY+3       ; get the missile's Y position

lsr                 ; divide by 2 for 2LK

sta Temp            ; save for position calculation

; Missile0Draw = ARENA_HEIGHT + BOX_HEIGHT - Y position

lda #(ARENA_HEIGHT + BOX_HEIGHT)

sec

sbc Temp

sta Missile1Draw

The routines are a little simpler than the others (for player0, player1 and ball) as the missiles don't have a Vertical Delay to set up. This is how the Arena looks with all the boxes drawn:

Not exactly right, is it? One thing we forgot to do was set the width of the missiles! We'd done that for the ball back in step 9 when we set the upper nybble of CTRLPF to 3:

TimerBar:

...

lda Variation       ; 3 20

lsr                 ; 2 22 - which Arena to show

tay                 ; 2 24 - set for index

ldx ArenaOffset,y   ; 4 28 - set X for which arena to draw

lda ArenaPF0,x      ; 4 32 - reflect and priority for playfield

and #%00000111      ; 2 34 - get the lower 3 bits for CTRLPF

ora #%00110000      ; 2 36 - set ball to display as 8x pixel

sta CTRLPF          ; 3 39

Which is why the ball was the right width when we added it in step 11. The same setting is needed for the missiles, but it goes into NUSIZ0 and NUSIZ1 instead:

VerticalSync:

...

lda #$30    ;

sta NUSIZ0  ; set missile0 to be 8x

sta NUSIZ1  ; set missile1 to be 8x

...

And now it looks correct:

OverScan was updated to detect collisions with the boxes drawn by the missiles:

OverScan:

...

notP0BL:

bit CXM0P       ; V=player0/missile0

bvc notP0M0     ; if V is off then player0 did not collide with missile0

ldx #2          ; which box was collected

jsr CollectBox  ; update score and reposition box

notP0M0:

bit CXM1P       ; N=player0/missile1

bpl notP0M1     ; if N is off then player0 did not collide with missile1

ldx #3          ; which box was collected

jsr CollectBox  ; update score and reposition box

notP0M1:

...

notP1BL:

bit CXM0P       ; N=player1/missile0

bpl notP1M0     ; if N is off then player1 did not collide with missile0

ldx #2          ; which box was collected

jsr CollectBox  ; update score and reposition box

notP1M0:

bit CXM1P       ; V=player1/missile1

bvc notP1M1     ; if V is off then player1 did not collide with missile1

ldx #3          ; which box was collected

jsr CollectBox  ; update score and reposition box

notP1M1:

A test run of the players collecting the boxes:

Lastly, on the off chance that somebody might roll the score, a new Digit graphic has been added:

And a minor change was made to CollectBox to display it and end the game if a player collected 100 boxes.

CollectBox:

sed                 ; SEt Decimal flag

clc                 ; CLear Carry bit

lda #1              ; 1 point per box

adc Score,y         ; add to player's current score

bcc Not100          ; if the Carry is clear, score did not roll

sta GameState       ; stop the game (A holds 0)

lda #$BB            ; B image is !! to show that score rolled

Not100:

sta Score,y         ; and save it

cld                 ; CLear Decimal flag

jsr RandomLocation  ; move box to new location

rts

To test it, I started up a game and used Stella's debugger to change the score to 99:

Then collected another box:

# Step 13: Add Sound Effects

A quiet game is playable, but it's more fun with sound. The TIA produces 2 channel sound. The channels are known as channel 0 and channel 1. There are 3 registers for each channel to control the sound produced:

- AUDC0, AUDC1 - Control of the channel, specifies the type of sound to generate. Values range from 0 to 15, though some values produce the same type of sound as others.

- AUDF0, AUDF1 - Frequency of the channel, values range from 0 (highest) to 31 (lowest).

- AUDV0, AUDV1 - Volume of the channel, values range from 0 (min) to 15 (max).

It's common to have a hand full of routines that handle sound effects. For Collect, these routines can be found in the new source code file sfx.asm. To add the file to collect.asm we use the include command:

include sfx.asm

Besides the file, we also need to allocate 2 RAM variables to keep track of the sound effects:

; indexes for sound effect driver

SFX_LEFT:       ds 1    ; stored in $B1

SFX_RIGHT:      ds 1    ; stored in $B2

The routines in sfx.asm are:

- SFX_UPDATE - Must be called once every frame. This routine updates the 6 TIA registers listed above.

- SFX_TRIGGER - must be called when there's a sound effect to generate, such as when an object is collected by the player. Use the Y register to denote which effect to trigger.

- SFX_OFF - routine that silences the sound effects. If not required by your game, it can be commented out to save 15 bytes of ROM.

sfx.asm also includes a couple data tables that SFX_UPDATE uses to update the 6 TIA registers. Yes, only 2 tables even though there are 3 registers per channel to update. AUDCx and AUDVx both only require a nybbles worth of data, so their information is combined into a single byte to save ROM.

The tables are defined like this for Collect:

SFX_F:

.byte 0, 31 ; collide

.byte 0,  0,  0,  0,  1,  1,  1,  2,  2,  2,  3,  3,  3 ; collect

.byte 0,  8,  8,  8,  8,  8,  8,  8,  8,  8,  8,  8,  8,  8,  8,  8 ; ping

.byte 0, 31, 31, 31, 31, 31, 31, 31, 31, 31, 31, 31, 31, 31, 31, 31 ; game over

SFX_CV:

.byte 0,$8f ; collide

sfxCOLLIDE = *-SFX_CV-1

.byte 0,$6f,$6f,$6f,$6f,$6f,$6f,$6f,$6f,$6f,$6f,$6f,$6f ; collect

sfxCOLLECT = *-SFX_CV-1

.byte 0,$41,$42,$43,$44,$45,$46,$47,$48,$49,$4a,$4b,$4c,$4d,$4e,$4f ; ping

sfxPING = *-SFX_CV-1

.byte 0,$cf,$cf,$cf,$cf,$cf,$cf,$cf,$cf,$cf,$cf,$cf,$cf,$cf,$cf,$cf ; game over

sfxGAMEOVER = *-SFX_CV-1

Each .byte line contains the data for a single sound effect. The two tables are used together, so data in the first .byte line of SFX_F goes along with the data in the first .byte line in SFX_CV. The number of values must be the same in each table and each .byte line. The first value in each .byte line should be 0, it denotes end-of-sfx (though if you have a long-duration sound effect you could span it over multiple .byte lines).

Table SFX_CV looks a little complicated because of the extra lines such as sfxPING = *-SFX_CV-1. All those are doing is calculating the value to be used when you trigger a sound effect. You can name your sound effects whatever you want, just make sure it's followed by = *-SFX_CV-1 (also make sure you have a space before and after the equal sign).

Lets look at a single sound effect (that's been slightly changed from above for clarity) to explain how the data is used:

SFX_F:

.byte 0,  1,  2,  3,  4,  5,  6,  7,  8,  9, 10, 11, 12 ; collect

SFX_CV:

.byte 0,$64,$65,$66,$67,$68,$69,$6a,$6b,$6c,$6d,$6e,$6f ; collect

sfxCOLLECT = *-SFX_CV-1

To trigger the sound effect, you'll use this in your code:

ldy #sfxCOLLECT     ; select sound effect

jsr SFX_TRIGGER     ; and trigger it

By triggering the sound effect, the value of sfxCOLLECT will be stored in one of the RAM variables. The value of sfxCOLLECT points to the 12 in the SFX_F table and the $6f in the SFX_CF table. When SFX_UPDATE is called (via jsr SFX_UPDATE), the 12 goes into AUDFx while the $6f will be split into two parts with the $6 going into AUDCx and the $f going into AUDFx. Lastly the pointer will be updated so it now points to 11 and $6e. After the next update they'll point to 10 and $6d, and so on until they point to 0 which means the end of the sound effect.

The order of the sound effects, as listed in the tables, is used to denote priority. The first sound effect has the lowest priority, so if both channels are busy and you try to trigger sound effect sfxCOLLIDE, nothing will happen. If both are busy and you try to trigger sfxGAMEOVER, the last sound effect, then one of the current sound effects will be aborted so sfxGAMEOVER can be heard.

SFX_UPDATE is called during Overscan:

OverScan:

sta WSYNC       ; Wait for SYNC (halts CPU until end of scanline)

lda #2          ; LoaD Accumulator with 2 so D1=1

sta VBLANK      ; STore Accumulator to VBLANK, D1=1 turns image output off

; set the timer so the total number of scanlines ends up being 262

lda #35

sta TIM64T

jsr SFX_UPDATE      ; update sound effects

While SFX_TRIGGER is called from a number of locations throughout the program. One of them is a tick sound that plays during the last 8 ticks of the timer:

DecrementTimer:

lsr TimerPF+5   ; PF2 right side, reversed bits so shift right

rol TimerPF+4   ; PF1 right side, normal bits so shift left

ror TimerPF+3   ; PF0 right side, reversed bits so shift right

lda TimerPF+3   ; only upper nybble used, so we need to put bit 3 into C

lsr

lsr

lsr

lsr

ror TimerPF+2   ; PF2 left side, reversed bits so shift right

rol TimerPF+1   ; PF1 left side, normal bits so shift left

ror TimerPF     ; PF0 left side, reversed bits so shift right

lda TimerPF+1   ; PF1 from left side

and #%00011111  ; check the lower 5 bits

bne NoTickSfx   ; branch if there's a value in the lower 5 bits

ldy #sfxPING    ; else do a sound effect

jsr SFX_TRIGGER

NoTickSfx:

In playing the game, I noticed it was harder to locate the box drawn by the ball, because it is the same color as the playfield, so I modified the program so it scores 2 points instead of just 1.

TestCollisions:

...

bit CXP0FB          ; N=player0/playfield, V=player0/ball

bvc notP0BL         ; if V is off, then player0 did not collide with ball

ldy #0              ; which score to update

ldx #4              ; which box was collected

jsr Collect2ptBox   ; update score and reposition box

...

bit CXP1FB          ; N=player1/playfield, V=player1/ball

bvc notP1BL         ; if V is off, then player1 did not collide with ball

ldy #1              ; which score to update

ldx #4              ; which box was collected

jsr Collect2ptBox   ; update score and reposition box

Collect2ptBox is a new routine that falls into CollectBox:

Collect2ptBox:

lda #2              ; 2 point box

.byte $2C           ; BIT with absolute addressing, trick that

; causes the lda #1 to be skipped over

CollectBox:

lda #1              ; 1 point per box

sed                 ; SEt Decimal flag

clc                 ; CLear Carry bit

adc Score,y         ; add to player's current score

# Step 14: Add Animation

Static images that just slide around the screen work OK, but we can do better—so for this update we'll add a couple new images so we can animate the players as they run around the arena.

While you can have as many frames of animation as you'd like, the code is most efficient if the number of frames is a power of 2 (2, 4, 8, 16, etc). The code that cycles through 4 frames is this:

Example:

inc frame

lda frame

and #3       ; limits values from 0-3, if A was 4 it becomes 0 after the and

sta frame

Just change the #3 to #7 (8 frames), #15 (16 frames) and so on. If you wanted to cycle through a non-power of 2 count, say for example 5 frames, the code would look like this:

Example:

ldx frame

inx

cpy #5

bne save

ldx #0

save:   stx frame

For Collect we're going to use 4 frames. You might be wondering why there's only 3 humanoid images—it's because we're going to use 1 of the images twice:

- HumanGfx

- HumanRunning0

- HumanRunning1

- HumanRunning0

To animate the players, we'll need to keep track of which frame they're showing, so let's add 2 new RAM variables:

; indexes for player animation sequences

Animation0:     ds 1    ; stored in $B3

Animation1:     ds 1    ; stored in $B4

Then modify PositionObjects so it will animate the images when it preps the variables for the 2LK, but only when the player is in motion:

PositionObjects:

...

; select image to show for Player0

lda ObjectX         ; get current X location for player 0

cmp SavedX          ;   compare with prior X location

bne Animate0        ;   if different, animate player 0

lda ObjectY         ; otherwise check current Y location

cmp SavedY          ;   against prior Y location

bne Animate0        ;   and animate player 0 if they're different

lda #0              ; if X and Y didn't change then select 0, the

beq SaveFrame0      ;   stationary image, and save it

Animate0:

inc Animation0      ; increment to select the next frame

lda Animation0      ; load it

and #3              ; limit to 0-3 (if it was 4, it's now 0)

SaveFrame0:

sta Animation0      ; save it

tax                 ; Transfer A to X

; Player0Ptr = HumanGfx + HUMAN_HEIGHT - 1 - Y position

lda ShapePtrLow,x   ; select image as specified in X

sec

sbc Temp

sta Player0Ptr

lda ShapePtrHi,x    ; select image as specified in X

sbc #0

sta Player0Ptr+1

...

ShapePtrLow:

.byte <(HumanGfx + HUMAN_HEIGHT - 1)

.byte <(HumanRunning0 + HUMAN_HEIGHT - 1)

.byte <(HumanRunning1 + HUMAN_HEIGHT - 1)

.byte <(HumanRunning0 + HUMAN_HEIGHT - 1)

.byte <(BoxGfx + HUMAN_HEIGHT - 1)

ShapePtrHi:

.byte >(HumanGfx + HUMAN_HEIGHT - 1)

.byte >(HumanRunning0 + HUMAN_HEIGHT - 1)

.byte >(HumanRunning1 + HUMAN_HEIGHT - 1)

.byte >(HumanRunning0 + HUMAN_HEIGHT - 1)

.byte >(BoxGfx + HUMAN_HEIGHT - 1)

The code for player 1 is almost the same, though it adds a test so the box image will be displayed for one player game variations:

; select image to show for Player1

bit Players

bpl UseBoxImage     ; if 1 player game then draw the box

lda ObjectX+1       ; get current X location for player 1

cmp SavedX+1        ;   compare with prior X location

bne Animate1        ;   if different, animate player 1

lda ObjectY+1       ; otherwise check current Y location

cmp SavedY+1        ;   against prior Y location

bne Animate1        ;   and animate player 1 if they're different

lda #0              ; if X and Y didn't change then select 0, the

beq SaveFrame1      ;   stationary image, and save it

Animate1:

inc Animation1      ; increment to select the next frame

lda Animation1      ; load it

and #3              ; limit to 0-3 (if it was 4, it's now 0)

SaveFrame1:

sta Animation1      ; save it

tax                 ; Transfer A to X

.byte $2C           ; $2C = BIT with absolute addressing, trick that

; causes the ldx #4 to be skipped over

UseBoxImage:

ldx #4              ; select the Box Image

; Player1Ptr = BoxGfx + HUMAN_HEIGHT - 1 - Y position

lda ShapePtrLow,x   ; select image as specified in X

sec

sbc Temp

sta Player1Ptr

lda ShapePtrHi,x    ; select image as specified in X

sbc #0

sta Player1Ptr+1

It works, but the players move so fast they look Superman running on a greased treadmill. See collect_20140714_toofast.bin in my blog entry. To fix that, we'll revise it to use an image over multiple frames. For testing, we'll make the left player use each image for 2 frames while the right uses each image for 4:

Animate0:

inc Animation0      ; increment to select the next frame

lda Animation0      ; load it

and #7              ; limit to 0-7 (if it was 8, it's now 0)

SaveFrame0:

sta Animation0      ; save it

lsr                 ; divide by 2 for 0-3 - this means we show the same

; image twice in succession

tax                 ; Transfer A to X

...

Animate1:

inc Animation1      ; increment to select the next frame

lda Animation1      ; load it

and #15             ; limit to 0-15 (if it was 16, it's now 0)

SaveFrame1:

sta Animation1      ; save it

lsr                 ; divide by 4 for 0-3 - this means we show the same

lsr                 ; image four times in succession

tax                 ; Transfer A to X

See collect_20140714_speedtest.bin in my blog entry.

Both look OK, though I think the left player looks a little better, so the final version will use each image twice. One minor thing happens when the game is over—if the players were in motion, the animation keeps on going even though the players are no longer in motion.

To fix that, we'll add a Game Over check (same logic was added for Player1) that will select the stationary image:

; select image to show for Player0

bit GameState

bpl StopAnimation0  ; if game is inactive, stop animation

lda ObjectX         ; get current X location for player 0

cmp SavedX          ;   compare with prior X location

bne Animate0        ;   if different, animate player 0

lda ObjectY         ; otherwise check current Y location

cmp SavedY          ;   against prior Y location

bne Animate0        ;   and animate player 0 if they're different

StopAnimation0:

lda #0              ; if X and Y didn't change then select 0, the

beq SaveFrame0      ;   stationary image, and save it